SDR Positioning System

Complete Technical Specification

Module structure · AGC · Attenuation · Trilateration · Kalman  |  2026-03-28

# 1. Module Structure

The entire project is a single Python package: sdr_positioning/. Nothing outside it needs to know about SoapySDR, gain values, or signal physics.

sdr_positioning/

__init__.py              # public API: PositioningSystem

sdr_module/              # hardware abstraction

__init__.py          # public: SDRModule, Measurement, CatalogueEntry

catalogue.py         # CatalogueLoader

receiver.py          # SDRReceiver (SoapySDR wrapper)

agc.py               # GainController

trilateration.py         # attenuation model + trilateration solver

kalman.py                # KalmanFilter + coordinate helpers

fusion.py                # FusionEngine (owns Kalman + sources)

models.py                # shared dataclasses: Measurement, PositionEstimate

stations.json            # signal catalogue (ships with module)

Dependency rule — imports flow in one direction only:

| File | May import from |
| --- | --- |
| sdr_module/* | Standard library + numpy + SoapySDR only |
| trilateration.py | sdr_module, models, numpy, scipy |
| kalman.py | models, numpy |
| fusion.py | trilateration, kalman, models, sdr_module |
| __init__.py | fusion (exposes PositioningSystem only) |

models.py exists to break circular imports. All shared dataclasses live there. No file imports from fusion.py except __init__.py.

External usage — the entire system in 5 lines:

from sdr_positioning import PositioningSystem

ps = PositioningSystem("sdr_positioning/stations.json")

ps.feed_imu(ax=0.1, ay=0.0, heading_deg=45., dt=0.02)

estimate = ps.step()   # returns PositionEstimate | None

# 2. Shared Dataclasses (models.py)

from dataclasses import dataclass, field

import time

@dataclass

class Measurement:

source_id:    str

rssi_dbm:     float   # normalised: measured_power - gain_used

freq_hz:      float

signal_type:  str

lat:          float   # transmitter position

lon:          float

power_w:      float   # transmitter ERP in watts

gain_used:    float   # dB — for diagnostics

antenna_gain_dbi: float = 0.0

best_effort:  bool  = False

timestamp:    float = field(default_factory=time.time)

@dataclass

class PositionEstimate:

lat:          float

lon:          float

accuracy_m:   float   # 1-sigma radius

speed_ms:     float

heading_deg:  float

source:       str     # "IMU" | "RF_UPDATE"

n_rf_sources: int

last_rf_age:  float   # seconds since last RF update

timestamp:    float = field(default_factory=time.time)

Measurement carries the transmitter lat/lon and power_w so that trilateration.py does not need to re-query the catalogue. The catalogue lookup happens once in sdr_module, not twice.

# 3. Signal Catalogue (catalogue.py)

## 3.1 JSON Schema

The existing station JSON is the canonical format. Key is frequency in MHz as string:

{

"91.4": {

"lat": 42.035704,  "lon": 23.095343,

"station": "РРС Благоевград",

"name": "БГ Радио",

"type": "FM",

"power_w": 300

},

"506.0": {

"lat": 42.6635,  "lon": 23.2946,

"station": "Витоша предавател",

"name": "БНТ 1",

"type": "DVB-T",

"power_w": 10000,

"antenna_gain_dbi": 6.0

}

}

## 3.2 Type Defaults Registry

| type | freq range MHz |
| --- | --- |
| FM | 87.5–108 |
| VOR | 108–118 |
| DAB | 174–240 |
| DVB-T | 470–862 |
| GSM | 935–960 |

## 3.3 CatalogueEntry Dataclass

@dataclass(frozen=True)

class CatalogueEntry:

source_id:        str    # e.g. "FM_91.4_Blagoevgrad"

freq_hz:          float  # "91.4" → 91.4e6

lat:              float

lon:              float

station:          str

name:             str

signal_type:      str

power_w:          float

antenna_gain_dbi: float

min_rssi_dbm:     float

max_rssi_dbm:     float = -20.0

# 4. SDR Module (sdr_module/)

## 4.1 SDRReceiver

Thin SoapySDR wrapper. The only file in the project that imports SoapySDR.

class SDRReceiver:

def set_gain(self, gain_db: float) -> None

def set_freq(self, freq_hz: float) -> None

def set_sample_rate(self, rate_hz: float) -> None

def read_power_dbm(self, n_samples=4096) -> float

# IQ capture → Hanning window → FFT → mean power in dBm

def close(self) -> None

## 4.2 Adaptive Gain Control

The AGC produces calibrated RSSI readings by finding the optimal gain for each signal type and normalising out the gain applied.

### Normalisation — the one formula

true_power_dbm = measured_power_dbm − gain_db

measured_power_dbm is what read_power_dbm() returns. gain_db is what was set on the receiver. true_power_dbm is what Measurement.rssi_dbm contains. This formula appears exactly once in GainController.measure().

### Per-type gain strategy

| Signal type | Target window (dBm) |
| --- | --- |
| FM | −55 to −75 |
| VOR | −65 to −85 |
| DAB | −60 to −80 |
| DVB-T | −55 to −75 |
| GSM | −35 to −55 |

Algorithm: set start_gain, read power, compute true_power = measured - gain. If true_power is outside the target window, adjust gain by (window_midpoint - measured) and retry. Max 4 steps. Return best-effort result if gain never fully settles.

Gain is reset to start_gain between signal types. GSM at gain=20 must not bleed into VOR which needs gain=70.

## 4.3 SDRModule Public API

class SDRModule:

def __init__(self, catalogue_path, driver="sdrplay", serial=None)

def scan(self, types=None) -> list[Measurement]

# Scans catalogue entries, returns normalised Measurements

# Measurement already contains transmitter lat/lon/power_w

def close(self)

# 5. Attenuation Model (trilateration.py)

Converts Measurement.rssi_dbm into an estimated distance in metres. Every physical parameter is explicit.

## 5.1 Free-Space Path Loss

The received power at distance d from a transmitter is:

RSSI_pred = EIRP_dbm − FSPL_dB

EIRP_dbm  = 10·log10(power_w) + 30 + antenna_gain_dbi

FSPL_dB   = 20·log10(d) + 20·log10(freq_hz) − 147.55

The constant −147.55 = 20·log10(4π/c) where c = 3×10⁸ m/s. It embeds wavelength: λ = c/f, so higher frequency = shorter wavelength = more FSPL. No separate wavelength correction is needed.

## 5.2 Inverting for Distance

path_loss_dB = EIRP_dbm − rssi_dbm

d_freespace  = 10^((path_loss_dB + 147.55 − 20·log10(freq_hz)) / 20)

## 5.3 Frequency Effect Summary

| Signal type | Frequency |
| --- | --- |
| FM | 100 MHz |
| VOR | 113 MHz |
| DAB | 200 MHz |
| DVB-T | 650 MHz |
| GSM | 948 MHz |

This is why GSM needs lower gain than FM even at the same distance — the path loss model accounts for this automatically via 20·log10(freq_hz).

# 6. Obstruction Compensation (trilateration.py)

## 6.1 Environment Classes

| Class | Description |
| --- | --- |
| OUTDOOR_LOS | Clear line of sight |
| OUTDOOR_URBAN | Outdoors, buildings present |
| INDOOR_LIGHT | Inside building, one exterior wall |
| INDOOR_DEEP | Multiple walls or floors |

## 6.2 Corrected Distance Formula

Obstruction adds extra loss on top of FSPL. Compensate by subtracting extra_loss_dB from the measured path loss before inverting:

path_loss_corrected = path_loss_dB − extra_loss_dB

d = 10^((path_loss_corrected + 147.55 − 20·log10(freq_hz)) / 20)

Without this correction, an indoor receiver would appear further from all transmitters than it actually is, because the walls are adding unaccounted loss.

## 6.3 Auto-Detection

Run one pass with OUTDOOR_LOS. Compute excess loss per source:

excess_loss = RSSI_predicted_at_d_freespace − rssi_dbm_measured

| Max excess_loss across sources | Detected environment |
| --- | --- |
| < 3 dB | OUTDOOR_LOS |
| 3–12 dB | OUTDOOR_URBAN |
| 12–22 dB | INDOOR_LIGHT |
| > 22 dB | INDOOR_DEEP |

Re-run with detected class. Two passes total. The environment class also inflates accuracy_m output (see Section 7.4).

## 6.4 RSSI Quality Filter

- rssi_dbm < entry.min_rssi_dbm → discard (noise floor)

- rssi_dbm > −20 dBm → discard (near-field, model invalid)

- best_effort = True → weight reduced by 0.5 in trilateration

- Fewer than 3 valid sources after filtering → return None

# 7. Trilateration Solver (trilateration.py)

## 7.1 Coordinate System

All solver coordinates are in local East-North metres relative to the centroid of the source positions. Convert from lat/lon using:

px = (lon − lon0) × 111320 × cos(radians(lat0))

py = (lat − lat0) × 110540

## 7.2 Objective Function

For N sources at known positions (xi, yi) with estimated distances di and weights wi, find (px, py) minimising:

E(px,py) = Σ wi · (sqrt((px−xi)² + (py−yi)²) − di)²

## 7.3 Source Weighting

Weight each source by inverse variance of its distance estimate. Use relative uncertainty κ = 0.3 (30% of estimated distance):

σi = di · κ

wi = 1 / σi²  =  1 / (di · 0.3)²

best_effort sources get wi × 0.5. This is the only place where source quality affects the solver — not an if-else chain, just a weight multiplier.

## 7.4 Solver

from scipy.optimize import minimize

result = minimize(

objective, x0=[cx, cy],

args=(sources, weights),

method="Nelder-Mead",

options={"xatol": 1.0, "fatol": 1.0, "maxiter": 500}

)

px, py = result.x

accuracy_m = sqrt(result.fun / N)

## 7.5 Geometry Check (GDOP)

Reject degenerate geometry where all sources cluster in one direction. Compute the largest angular gap between sources as seen from the estimated position:

angles   = [atan2(s.x - px, s.y - py) for s in sources]

max_gap  = max angular gap including wrap-around

if max_gap > 270 degrees:  # sources span < 90 degrees

accuracy_m *= 3.0       # inflate, do not discard

## 7.6 Environment Accuracy Inflation

| Environment class | accuracy_m multiplier |
| --- | --- |
| OUTDOOR_LOS | 1.0× |
| OUTDOOR_URBAN | 1.3× |
| INDOOR_LIGHT | 2.0× |
| INDOOR_DEEP | 3.0× |

This multiplier is applied after the solver residual. It reflects that indoor ranging is less reliable even after compensation.

## 7.7 Public Function

def trilaterate(

measurements: list[Measurement],

environment: Environment = Environment.OUTDOOR_LOS,

origin: tuple[float,float] | None = None

) -> RFMeasurement | None:

# Stage 1: quality filter

# Stage 2: auto-detect environment (2 passes)

# Stage 3: RSSI → distance with obstruction compensation

# Stage 4: weighted Nelder-Mead solve

# Stage 5: geometry check, accuracy inflation

# Returns RFMeasurement(lat, lon, accuracy_m) or None

# 8. Kalman Filter (kalman.py)

## 8.1 State Vector

x = [px, py, vx, vy]ᵀ   (metres and m/s in local ENU frame)

Acceleration is not in the state — it arrives from the IMU each step and is applied via the control input matrix B.

## 8.2 Predict Step

IMU delivers pre-processed (ax_body, ay_body, heading_deg, dt). FusionEngine rotates to world frame before calling predict():

ax_world = ax_body·sin(ψ) + ay_body·cos(ψ)

ay_world = ax_body·cos(ψ) − ay_body·sin(ψ)

State transition:

x̂ = F·x̂ + B·[ax_world, ay_world]ᵀ

F = [[1,0,dt,0],[0,1,0,dt],[0,0,1,0],[0,0,0,1]]

B = [[dt²/2,0],[0,dt²/2],[dt,0],[0,dt]]

P = F·P·Fᵀ + Q

Process noise Q uses σ_a = 0.1 m/s² (tunable constructor argument).

## 8.3 Update Step

Fires when a valid RF fix arrives. z = [px_rf, py_rf]ᵀ in local ENU metres. R = diag(accuracy_m², accuracy_m²).

S = H·P·Hᵀ + R

K = P·Hᵀ·S⁻¹

x̂ = x̂ + K·(z − H·x̂)

P = (I − K·H)·P

Outlier gate: reject fix if Mahalanobis distance d² = (z−Hx̂)ᵀ·S⁻¹·(z−Hx̂) > 5.991 (χ²₀.₉₅,₂). Widen to 13.816 during first 10 seconds.

# 9. FusionEngine (fusion.py)

class FusionEngine:

def __init__(self, sdr: SDRModule, sigma_a=0.1)

def feed_imu(self, ax_body, ay_body, heading_deg, dt) -> None

# Rotates to world frame, calls kf.predict()

def step(self) -> PositionEstimate | None

# 1. sdr.scan() → measurements

# 2. trilaterate(measurements) → RFMeasurement | None

# 3. If fix: set origin on first fix, kf.update()

# 4. Return PositionEstimate from current kf state

## 9.1 PositioningSystem (public facade)

# sdr_positioning/__init__.py

class PositioningSystem:

def __init__(self, catalogue_path, driver="sdrplay", serial=None, sigma_a=0.1):

sdr = SDRModule(catalogue_path, driver, serial)

self._engine = FusionEngine(sdr, sigma_a)

def feed_imu(self, ax, ay, heading_deg, dt): ...

def step(self) -> PositionEstimate | None: ...

def close(self): ...

# 10. Implementation Checkpoints

Build in order. Each checkpoint is independently runnable. Do not proceed until the test passes.

Checkpoint 1 — models.py and catalogue loading

Create models.py with Measurement and PositionEstimate. Create CatalogueLoader. Verify the existing stations.json loads correctly.

### Test

from sdr_positioning.sdr_module.catalogue import CatalogueLoader

entries = CatalogueLoader().load("sdr_positioning/stations.json")

fm = [e for e in entries if e.signal_type == "FM"]

assert all(e.freq_hz > 1e6 for e in fm),  "freq must be Hz not MHz"

assert all(e.power_w > 0 for e in entries)

assert all(e.min_rssi_dbm < 0 for e in entries)

assert all(e.max_rssi_dbm == -20. for e in entries)

# Type defaults applied

assert fm[0].antenna_gain_dbi == 0.0

# DVB-T default gain

dvbt = [e for e in entries if e.signal_type == "DVB-T"]

if dvbt: assert dvbt[0].antenna_gain_dbi == 6.0

Checkpoint 2 — Attenuation model — rssi_to_distance()

Implement rssi_to_distance() in trilateration.py. Test round-trip accuracy and obstruction compensation.

### Test

from sdr_positioning.trilateration import rssi_to_distance, Environment

from math import log10, cos, radians

# Synthetic: 50 kW FM at 103.2 MHz, receiver 10 km away, outdoor LOS

power_w, freq_hz, d_true = 50000., 103.2e6, 10000.

eirp_dbm = 10*log10(power_w) + 30 + 0.0

fspl     = 20*log10(d_true) + 20*log10(freq_hz) - 147.55

rssi_pred = eirp_dbm - fspl

d_est = rssi_to_distance(rssi_pred, freq_hz, power_w, 0.0, Environment.OUTDOOR_LOS)

assert abs(d_est - d_true) < 1., "round-trip must recover true distance"

# Indoor compensation: 15 dB extra loss, INDOOR_LIGHT corrects it

d_indoor = rssi_to_distance(rssi_pred - 15., freq_hz, power_w, 0.0, Environment.INDOOR_LIGHT)

assert abs(d_indoor - d_true) < 100., "indoor correction must approximately recover"

# Frequency effect: GSM 948 MHz should give shorter apparent distance

# than FM 103 MHz at same true RSSI (more path loss at higher freq)

d_gsm = rssi_to_distance(rssi_pred, 948e6, power_w, 0.0, Environment.OUTDOOR_LOS)

assert d_gsm < d_true, "GSM higher freq must imply shorter apparent distance"

Checkpoint 3 — Trilateration solver

Implement the Nelder-Mead solver and geometry check. Test with synthetic data at known position.

### Test

import numpy as np

from sdr_positioning.trilateration import trilaterate_solver, geometry_ok

true_px, true_py = 500., 300.

source_positions = [(0.,0.), (1000.,0.), (1000.,1000.), (0.,1000.)]

np.random.seed(42)

sources = []

for sx, sy in source_positions:

d = np.sqrt((true_px-sx)**2 + (true_py-sy)**2)

sources.append({"x":sx, "y":sy, "d_est": d + np.random.normal(0, d*0.1)})

(px, py), residual = trilaterate_solver(sources)

error = np.sqrt((px-true_px)**2 + (py-true_py)**2)

assert error < 100., f"error {error:.1f}m must be under 100m"

assert residual >= 0.

# Poor geometry — sources clustered

bad = [{"x":900.,"y":250.,"d_est":400.},

{"x":950.,"y":300.,"d_est":450.},

{"x":980.,"y":310.,"d_est":490.}]

assert not geometry_ok(bad, px, py), "clustered sources must fail"

Checkpoint 4 — AGC and SDRModule with mock hardware

Test GainController normalisation and the full scan() pipeline using a mock SDRReceiver.

### Test

from sdr_positioning.sdr_module.agc import GainController, GAIN_STRATEGIES

from sdr_positioning.sdr_module.catalogue import CatalogueLoader

class MockReceiver:

TRUE_POWER = -65.  # signal is -65 dBm at antenna

def __init__(self): self.gain = 0.

def set_gain(self, g): self.gain = g

def set_freq(self, f): pass

def set_sample_rate(self, r): pass

def read_power_dbm(self, n=4096): return self.TRUE_POWER + self.gain

entry = CatalogueLoader().load("sdr_positioning/stations.json")[0]

agc   = GainController()

m     = agc.measure(entry, MockReceiver())

assert m is not None

assert abs(m.rssi_dbm - (-65.)) < 0.5, "normalised RSSI must equal true power"

assert m.lat == entry.lat and m.lon == entry.lon, "transmitter coords must pass through"

# Saturation — should return None

class StrongRx(MockReceiver): TRUE_POWER = -10.

assert agc.measure(entry, StrongRx()) is None

# Noise floor — should return None

class WeakRx(MockReceiver): TRUE_POWER = -95.

assert agc.measure(entry, WeakRx()) is None

Checkpoint 5 — KalmanFilter

Test predict, update, and outlier rejection.

### Test

import numpy as np

from sdr_positioning.kalman import KalmanFilter

kf = KalmanFilter(sigma_a=0.1)

kf.x = np.array([100., 200., 0., 0.])

kf.P = np.diag([100., 100., 25., 25.])

# Static: zero accel, position must not drift

for _ in range(100): kf.predict(0., 0., 0.02)

assert abs(kf.x[0] - 100.) < 1e-6

assert kf.P[0,0] > 100., "uncertainty must grow"

# Update pulls toward measurement

kf.x = np.zeros(4); kf.P = np.diag([500.,500.,25.,25.])

ok = kf.update(px_rf=50., py_rf=50., accuracy_m=30.)

assert ok

assert kf.x[0] > 0. and kf.x[1] > 0.

assert kf.P[0,0] < 500.

# Outlier rejected

kf.P = np.diag([10.,10.,1.,1.])

assert not kf.update(9999., 9999., 30.), "outlier must be rejected"

Checkpoint 6 — Full integration smoke test

Wire all components together through PositioningSystem. Uses mock hardware throughout.

### Test

from unittest.mock import patch, MagicMock

from sdr_positioning import PositioningSystem

# Patch SoapySDR so no real hardware needed

with patch("sdr_positioning.sdr_module.receiver.SoapySDR") as mock_soapy:

mock_dev = MagicMock()

mock_soapy.Device.return_value = mock_dev

ps = PositioningSystem("sdr_positioning/stations.json")

# Inject mock power reading: -60 dBm at all frequencies

ps._engine.sdr.receiver.read_power_dbm = lambda n=4096: -60.

ps._engine.sdr.receiver.set_gain = lambda g: None

ps._engine.sdr.receiver.set_freq = lambda f: None

ps._engine.sdr.receiver.set_sample_rate = lambda r: None

# Before first fix — no estimate

assert ps.step() is None

# After RF fix available (≥3 FM sources in catalogue)

result = ps.step()

# May be None if fewer than 3 sources — acceptable in smoke test

# IMU feed must not crash

ps.feed_imu(ax=0.1, ay=0., heading_deg=45., dt=0.02)

result = ps.step()

if result is not None:

assert result.accuracy_m > 0.

assert -90 < result.lat < 90

assert -180 < result.lon < 180

ps.close()

print("Integration smoke test: OK")

# 11. Extension Points

| Extension | Where and how |
| --- | --- |
| New signal type | Add to GAIN_STRATEGIES in agc.py + TYPE_DEFAULTS in catalogue.py. Zero other changes. |
| New hardware | Implement SDRReceiver interface. Pass driver= to SDRModule. |
| Phone LTE cells (v2) | Add PhoneCellSource to sdr_module/ implementing same Measurement output. FusionEngine polls it transparently. |
| VOR bearing (EKF) | Add kf.update_bearing() to kalman.py. trilateration.py unchanged. |
| GPS cross-check | Add GPSValidator to fusion.py. Never integrates GPS — compares against kf.x and emits spoofing flag. |
| REST API output | Wrap PositioningSystem in FastAPI. PositionEstimate is already a dataclass — json.dumps(asdict(estimate)). |

# 12. Design Rules

- SoapySDR imported in exactly one file: receiver.py.

- true_power = measured - gain appears exactly once: GainController.measure().

- FSPL formula appears exactly once: rssi_to_distance() in trilateration.py.

- CatalogueEntry is frozen — immutable after load.

- Measurement.rssi_dbm is always normalised before leaving sdr_module/.

- Adding a signal type requires editing two dicts, zero if-else chains.

- scan() never raises — returns empty list if no signals found.

- trilaterate() never raises — returns None if insufficient sources.

- step() never raises — returns None if no fix available yet.

End of specification